# PORTABLE WATER FILTER AND SAFETY MONITORING SYSTEM

EMBEDDED SYSTEMS: COEPRO15

Submitted by:

Alon, Angel Andrein L.

Amaba, Ariel John A.

Barcelona, Leo Jay B.   De

Jesus, Christian Garcia, Llyod Andrei D.

Ilao, Kyle Andrei James Z.

Jovelo, Robie R.

Macabangon, Pia Nicole B.

Ortenero, Mel Paolo

Bachelor of Science in Computer Engineering

Submitted in the College of Engineering and Digital Technology

Department of Computer Engineering

Sir. Meiko Rios

Objectives

- To develop a system that detects water quality using pH, turbidity, TDS, and temperature sensors.

- To provide real-time safety feedback through display, LED indicators, and buzzer alerts.

- To ensure portability with rechargeable battery power management.

- To design a compact and user-friendly embedded system for quick water safety checks.

- To integrate a basic water filtration mechanism that improves water quality after detection.

# System Functions

- Water Quality Monitoring System: Uses pH, turbidity, TDS, and temperature sensors to analyze water samples.

- Threshold Detection: Categorizes readings into Safe, Warning, Unsafe based on WHO standards.

- Alert System: Activates LED indicators and buzzer for real-time notification.

- Display System: Shows sensor readings and safety status on OLED/LCD.

- Power Management: Provides portable, rechargeable energy via Li-ion battery and TP4056 module.

- Water Filtration Control System: Enables the system to initiate a filtration process when water quality is classified as unsafe or at a warning level, using a pump and filter setup to improve water condition before conducting another analysis.

| Technical Components |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category | Component | Quantity | Purpose |  |
| Microcontroller | Arduino Nano/Uno | 1 | Processes sensor signals and controls display |  |
| Sensor | pH Sensor Module | 1 | Detects acidity/alkalinity |  |
| Sensor | Turbidity Sensor Module | 1 | Measures water clarity |  |
| Sensor | TDS Sensor Module | 1 | Measures dissolved solids |  |
| Sensor | TemperatureSensor (DS18B20) | 1 | Measures water temperature |  |
| Display | OLED/LCD Displa | 1 | Shows readings and safety status |  |
| Output | LED Indicartors (Green, Yellow, Red) | 3 | Displays water safety status |  |
| Output | Active Piezo Buzzer | 1 | Produces alert sound |  |
| Power | V9 Battery | 2 | Portable power source |  |
| Interconnects | Wires | 1 | Conductive paths for electricity. |  |
| Actuator | Mini water pump activated water filter | 1 | Moves water through filtration system |  |
| Filter | Sendiment Filter | 1 | Removes impurities and odor |  |
| Control | Relay Control | 1 | Controls pump ON/OF |  |
| Tubin | Tubing | 1 Set | Water flow connection |  |

Prototype Visualization:

Prototype Description: The proposed prototype is a compact embedded device with sensors immersed in water samples. The system includes:

- Sensor probes for pH, turbidity, TDS, and temperature

- OLED/LCD display for real-time readings

- LED indicators and buzzer for alerts

- Rechargeable battery with TP4056 charging module

- Breadboard/PCB for connections

- 3-stage water filtration module (sediment, carbon, and UF/CB filter)

- Relay-controlled water pump for automated filtration process

Filtration and Water Flow Mechanism: The system is enhanced with a filtration module that activates when water quality is detected as unsafe or at a warning level. A relaycontrolled water pump directs the water through a multi-stage filtration system to remove impurities such as suspended particles, odor, and some microorganisms. After filtration, the water is re-tested by the sensors to verify improvement in quality.

Closed-Loop Feedback System: The prototype operates in a feedback loop where water is continuously monitored, filtered when necessary, and re-evaluated. This ensures more reliable and accurate water quality assessment.

Instant Safety Status Indicator: Simplifies interpretation with clear categories.

- Non-Invasive Measurement: Uses sensor-based technology for easy sampling.

- Portable and Lightweight Design: Compact, battery-powered, and easy to carry.

- User-Friendly Interface: Clear OLED/LCD display for accessibility.

- Low-Cost Implementation: Affordable components make it accessible to communities.

- Automated Threshold-Based Detection: Provides immediate safety classification.

- Energy-Efficient Operation: Optimized power management for extended use.

- Integrated Filtration System: Automatically filters water when unsafe levels are detected.

- Automated Water Flow Control: Utilizes a pump and relay module to regulate the filtration process.

| Schematic Connections: |  |  |
| --- | --- | --- |
| Component | Arduino Pin | Connection Type |
| pH Sensor | A0 | Analog Input |
| Turbidity Sensor | A1 | Analog Input |
| TDS Sensor | A2 | Analog Input |
| Temperature Sensor | D2 | Digital Input |
| OLED (SDA) | A4 | I2C Data Line |
| OLED (SCL) | A5 | I2C Clock Line |
| Green LED | D8 | Digital Output |
| Yellow LED | D9 | Digital Output |
| Red LED | D10 | Digital Output |
| Buzzer | D11 | Digital Output |
| V9 Battery | VIN/GND | Power Supply |
| Relay Module | D7 | Digital Output |
| Water Pump | External Power | Controlled by Relay |

| Estimated Cost (PHP): |  |
| --- | --- |
| Item | Estimated Price |
| Arduino Nano/Uno | ₱287.00 |
| OLED Display | ₱230.00 |
| pH Sensor | ₱582.00 |
| Turbidity Sensor | ₱290.00 |
| TDS Sensor | ₱516.00 |
| Temperature Sensor | ₱167.00 |
| 9V Energizer Battery | ₱100.00 |
| LEDs Indicators, Wires | ₱102.00 |
| Active Passive Buzzer | ₱105.00 |
| Mini Water Pump/filter | ₱116.00 |
| Relay Module | ₱128.00 |
| TOTAL: ₱2,623.00 |  |

COMPONENTS:

ARDUINO UNO

| SHOP NAME | PRICE |
| --- | --- |
| Magicc.ph | ₱ 287.00 |

TURBIDITY SENSOR MODULE

| SHOP NAME | PRICE |
| --- | --- |
| Astroc.ph | ₱ 290.00 |

TDS SENSOR MODULE

| SHOP NAME | PRICE |
| --- | --- |
| MakerLab | ₱ 561.00 |

TEMPERATURE SENSOR MODULE (DS18B20)

| SHOP NAME | PRICE |
| --- | --- |
| Saiyixing.ph | ₱ 167.00 |

OLED/LCD DISPLAY

| SHOP NAME | PRICE |
| --- | --- |
| Kingzizi.ph | ₱ 230.00 |

LED INDICATORS (YELLOW, GREEN, RED)

| SHOP NAME | PRICE |
| --- | --- |
| JB Electrical Supplies | ₱ 102.00 |

ACTIVE PASSIVE BUZZER

| SHOP NAME | PRICE |
| --- | --- |
| Lebito.ph | ₱ 105.00 |

9V BATTERY

| SHOP NAME | PRICE |
| --- | --- |
| Pandayan Plus | ₱ 107.00 |

MINI WATER PUMP

| SHOP NAME | PRICE |
| --- | --- |
| Mexus Tools | ₱ 116.00 |

RELAY MODULE

| SHOP NAME | PRICE |
| --- | --- |
| Zengchuangjian.ph | ₱ 128.00 |

PH SENSOR

| SHOP NAME | PRICE |
| --- | --- |
| Reland2.ph | ₱ 582.00 |